'Embedded Systems' EXPERIMENT 4

From

Name::M Husnain

Khalid

Roll No: 22l-6286

Instructor:Muhammad Ahmad

#include <stdio.h>

#include <stdint.h>

#include "C:\ti\TivaWare_C_Series-2.1.4.178\inc\tm4c123gh6pm.h"

void PWM1_Init(uint16_t period, uint16_t duty)

{

// 1. Enable clocks for PWM1 and Port F

SYSCTL_RCGCPWM_R |= 0x02;        // PWM Module 1 clock

SYSCTL_RCGCGPIO_R |= 0x20;       // Port F clock (bit 5)

while((SYSCTL_PRGPIO_R & 0x20) == 0);

// 2. Configure PF2 for PWM (M1PWM6)

GPIO_PORTF_AFSEL_R |= 0x04;      // Enable alternate function PF2

GPIO_PORTF_PCTL_R &= ~0x00000F00;

GPIO_PORTF_PCTL_R |= 0x00000500; // PF2 = M1PWM6

GPIO_PORTF_DEN_R |= (1<<2);

GPIO_PORTF_DIR_R |= (1<<2);

// 3. Use PWM clock divider

SYSCTL_RCC_R |= 0x00100000;        // Use PWM divider

SYSCTL_RCC_R = (SYSCTL_RCC_R & ~0x000E0000) | 0x00000000; // PWM clock = system clock

// 4. Disable generator during configuration

PWM1_3_CTL_R = 0;

// 5. Set PWM period and duty

PWM1_3_LOAD_R = period - 1;

PWM1_3_CMPA_R = duty - 1;

// 6. Generator actions: HIGH at reload, LOW at compare

PWM1_3_GENA_R = 0x8C;

// 7. Enable generator

PWM1_3_CTL_R |= 0x01;

//8. Do NOT enable PWM output here — controlled in main loop

PWM1_ENABLE_R |= (1<<6);

}

void PortD_Init(void)

{

SYSCTL_RCGCGPIO_R |= (1<<3);     // Enable Port D clock

while((SYSCTL_PRGPIO_R & (1<<3)) == 0);

// Unlock PD7

GPIO_PORTD_LOCK_R = 0x4C4F434B;

GPIO_PORTD_CR_R |= (1<<7);

// PD7 input with pull-up

GPIO_PORTD_DIR_R &= ~(1<<7);

GPIO_PORTD_DEN_R |= (1<<7);

GPIO_PORTD_PUR_R |= (1<<7) ;

}

int main(void)

{

// 1 kHz tone for 16 MHz clock

PWM1_Init(16000, 8000);

PortD_Init();

while(1)

{

// Button pressed ? PD7 = 0 (active LOW)

if((GPIO_PORTD_DATA_R & (1<<7)) == 0)

{

PWM1_ENABLE_R |= (1<<6);   // Buzzer ON

}

else

{

PWM1_ENABLE_R &= ~(1<<6);  // Buzzer OFF

}

}

}