'Embedded Systems' EXPERIMENT 11

USING INTERNAL COUNTERS WITH SYSTICK

TIMER AND PHASE LOCKED LOOP (PLL)

From

Name::M Husnain

Khalid

Roll No: 22l-6286

Instructor:Muhammad Ahmad

EXERCISE – 1:

Use systick timer to write a routine to add a delay of 10ms using main oscillator. Toggle PF2 every 10ms.Please give your code for solving the above question in the space below:

Code:

#include <stdio.h>

#include <stdint.h>

#include "C:\ti\TivaWare_C_Series-2.1.4.178\inc\tm4c123gh6pm.h"

#define PF2       0x04  // PF2 mask

// Port F Init (PF2 output)

void PortF_Init(void){

volatile uint32_t delay;

SYSCTL_RCGCGPIO_R |= 0x20;           // Port F clock

delay = SYSCTL_RCGCGPIO_R;

GPIO_PORTF_LOCK_R = 0x4C4F434B;      // Unlock

GPIO_PORTF_CR_R |= PF2;              // Commit

GPIO_PORTF_AMSEL_R &= ~PF2;          // Analog off

GPIO_PORTF_PCTL_R &= ~0x00000F00;    // GPIO mode

GPIO_PORTF_DIR_R |= PF2;             // Output

GPIO_PORTF_AFSEL_R &= ~PF2;          // No alt func

GPIO_PORTF_DEN_R |= PF2;             // Digital

}

// Main Oscillator: 16 MHz MOSC (PLL bypassed)

void MainOsc_Init(void){

SYSCTL_RCC2_R |=  0x80000000;        // RCC2

SYSCTL_RCC2_R |=  0x00000800;        // Bypass PLL

SYSCTL_RCC_R = (SYSCTL_RCC_R & ~0x000007C0) | 0x00000540;  // 16 MHz XTAL

SYSCTL_RCC2_R &= ~0x00000070;        // MOSC source

SYSCTL_RCC2_R = (SYSCTL_RCC2_R & ~0x1FC00000) | (0 << 22); // /1

}

// SysTick Init: 10 ms @ 16 MHz

void SysTick_Init(void){

NVIC_ST_CTRL_R = 0;

NVIC_ST_RELOAD_R = 160000-1;           // 160k cycles

NVIC_ST_CURRENT_R = 0;

NVIC_ST_CTRL_R = 0x00000005;         // SysClk + Enable

}

// 10 ms Delay

void SysTick_Wait10ms(void){

NVIC_ST_CURRENT_R = 0;

while((NVIC_ST_CTRL_R & 0x00010000) == 0){};

}

// Main

int main(void){

PortF_Init();

MainOsc_Init();

SysTick_Init();

while(1){

GPIO_PORTF_DATA_R ^= PF2;

SysTick_Wait10ms();

//SysTick_Wait10ms();

//SysTick_Wait10ms();

GPIO_PORTF_DATA_R &= ~(PF2);

SysTick_Wait10ms();

//SysTick_Wait10ms();

//SysTick_Wait10ms();

}

}