'Embedded Systems' EXPERIMENT 3

CONFIGURING GPIO PORTS OF TM4C123GH6PM FOR DIGITAL I/O

From

Name::M Husnain

Khalid

Roll No: 22l-6286

Instructor:Muhammad Ahmad

Exercise 1

#include <stdio.h>

#include <stdint.h>

#include "C:\ti\TivaWare_C_Series-2.1.4.178\inc\tm4c123gh6pm.h"

#define SW1 (GPIO_PORTF_DATA_R = *((volatile unsigned int *)0x40025040))

void PortF_Init(void){

SYSCTL_RCGCGPIO_R |= 0x00000020; // activate clock for Port F

while((SYSCTL_PRGPIO_R&0x00000020) == 0){};// ready?

GPIO_PORTF_LOCK_R = 0x4C4F434B; // unlock GPIO Port F

GPIO_PORTF_CR_R = 0x14; // allow changes to PF2 & PF4

GPIO_PORTF_DIR_R = 0x04; // PF4 in, PF2 out

GPIO_PORTF_PUR_R = 0x10; // enable pull-up on PF4

GPIO_PORTF_DEN_R = 0x14; // enable digital I/O on PF2 & PF4

}

void main(void)

{PortF_Init();

while(1){

if (!SW1)

GPIO_PORTF_DATA_R=GPIO_PORTF_DATA_R^0x04;

}

}

Exercise 2

#include <stdio.h>

#include <stdint.h>

#include "C:\ti\TivaWare_C_Series-2.1.4.178\inc\tm4c123gh6pm.h"

/*

PF1 - red LED

PF2 - blue LED

PF3 - green LED

*/

void delay()

{

uint32_t counter =0;

for (counter =0;counter<1500000;counter++);

}

int main(void)

{

volatile uint32_t j;

SYSCTL_RCGCGPIO_R |= 0x00000020; // activate clock for Port F

GPIO_PORTF_LOCK_R = 0x4C4F434B; // unlock GPIO Port F

GPIO_PORTF_CR_R = 0x0E; // allow changes to PF2 & PF4

GPIO_PORTF_DIR_R = 0x0E;

GPIO_PORTF_DEN_R = 0x0E;

while(1){

for(j=0;j<8;j++)

{

GPIO_PORTF_DATA_R = (j<<1);

delay();

}

}

}