Lab 6

Embedded System

From

Name::M Husnain

Khalid

Roll No: 22l-6286

Instructor:Muhammad

Ahmad

EXPERIMENT 5

INTERACTING WITH TM4C123GH6PM THROUGH TERMINAL

USING UART MODULE

EXERCISE:

Following are the functions that transmit and receive a single character to and from serial port.

void UART0Tx(char c)

{

while((UART0_FR_R & 0x0020) !=0);

UART0_DR_R = c;

}

char UART0Rx(void){

while ((UART0_FR_R&0x0010)!=0);

return ((char)(UART0_DR_R&0xFF));

}

Using UART0Tx(char) function, define another function void sendString(char*ptr) that sends a

string to serial port.

Using all these functions write a code that receives a character from PC and sends next character to

PC. Baud rate should be set to 115200.

The output on TeraTerm should be as given in screenshot.

Note: \n is used for new line character and \r is used for moving cursor to start of line

Please give your code for solving the above question in the space below and paste the

screenshot of the output:

CODE:

#include <stdint.h>

#include <stdio.h>

#include "C:\ti\TivaWare_C_Series-2.1.4.178\inc\tm4c123gh6pm.h"

void UART0Tx(char c) {

while((UART0_FR_R & 0x20) != 0);

UART0_DR_R = c;

}

void sendString(char *ptr) {

while (*ptr != '\0') {

UART0Tx(*ptr);

ptr++;

}

}

char UART0Rx(void) {

while((UART0_FR_R & 0x10) != 0);

return (char)(UART0_DR_R & 0xFF);

}

int main(void) {

SYSCTL_RCGCUART_R |= 0x01;

SYSCTL_RCGCGPIO_R |= 0x01;

UART0_CTL_R &= ~0x01;

UART0_IBRD_R = 8;

UART0_FBRD_R = 44;

UART0_LCRH_R = 0x70;

UART0_CTL_R = 0x301;

GPIO_PORTA_AFSEL_R |= 0x03;

GPIO_PORTA_PCTL_R |= 0x11;

GPIO_PORTA_DEN_R |= 0x03;

while(1) {

char c;

sendString("Enter the Character to send:");

c = UART0Rx();

UART0Tx(c );

sendString("\n\r");

sendString("Next Character is:");

UART0Tx(c + 1);

sendString("\n\r");

}

}