EXPERIMENT 6

From : M Husnain Khalid
22L:6286

To : Muhammad Ahmad

CONFIGURING MASTER-SLAVE COMMUNICATION

SYSTEM USING SYNCHRONOUS SERIAL INTERFACE

EXERCISE:

For the problems given below, consider the following scenario:

Two TM4C controllers are connected with each other via SPI. Assume both are using SSI2 module.

One is configured as a master and the other is slave. Master sends upper case letter to slave and

slave returns its lower case

Ans

Master Code:

#include <stdint.h>

#include "tm4c123gh6pm.h"

void SSI2_Init(void);

int main()

{

char data;

SSI2_Init();

data = 'A';

while(1)

{

while((SSI2_SR_R & 0x02) == 0);   // wait until TX FIFO not full

SSI2_DR_R = data;                 // send uppercase

while((SSI2_SR_R & 0x10) != 0);   // wait until not busy

while((SSI2_SR_R & 0x04) == 0);   // wait until data received

data = SSI2_DR_R;                 // receive lowercase

}

}

void SSI2_Init(void)

{

SYSCTL_RCGCSSI_R |= 0x04;     // enable SSI2 clock

SYSCTL_RCGCGPIO_R |= 0x02;    // enable PORTB clock

while((SYSCTL_PRGPIO_R & 0x02)==0);

GPIO_PORTB_AFSEL_R |= 0xF0;   // enable alternate function

GPIO_PORTB_PCTL_R |= 0x22220000;

GPIO_PORTB_DEN_R |= 0xF0;

SSI2_CR1_R = 0;               // disable SSI

SSI2_CC_R = 0;                // system clock

SSI2_CPSR_R = 10;

SSI2_CR0_R = 0x0007;          // 8-bit data, SPI mode

SSI2_CR1_R |= 0x02;           // master mode

SSI2_CR1_R |= 0x02;           // enable SSI

}

Slave Code:

#include <stdint.h>

#include "tm4c123gh6pm.h"

void SSI2_Init(void);

int main()

{

char data;

SSI2_Init();

while(1)

{

while((SSI2_SR_R & 0x04) == 0);   // wait receive

data = SSI2_DR_R;

data = data + 32;                 // convert to lowercase

while((SSI2_SR_R & 0x02) == 0);   // wait TX ready

SSI2_DR_R = data;

}

}

void SSI2_Init(void)

{

SYSCTL_RCGCSSI_R |= 0x04;

SYSCTL_RCGCGPIO_R |= 0x02;

while((SYSCTL_PRGPIO_R & 0x02)==0);

GPIO_PORTB_AFSEL_R |= 0xF0;

GPIO_PORTB_PCTL_R |= 0x22220000;

GPIO_PORTB_DEN_R |= 0xF0;

SSI2_CR1_R = 0;

SSI2_CC_R = 0;

SSI2_CPSR_R = 10;

SSI2_CR0_R = 0x0007;

SSI2_CR1_R = 0x04;   // slave mode

SSI2_CR1_R |= 0x02;  // enable SSI

}

In this experiment, the SSI (Synchronous Serial Interface) module of the TM4C microcontroller is configured to implement SPI communication between two controllers. The clocks for the SSI module and GPIO port are enabled and the required pins are configured for alternate SSI function. The SSI control registers are set to select 8-bit data, SPI frame format, and master/slave mode. Data transmission and reception are performed using the SSI data register, while the status register is used to monitor FIFO and busy flags. The master sends an uppercase character to the slave, and the slave converts it to lowercase and sends it back.