Embedded System

From

Name::M Husnain

Khalid

Roll No: 22l-6286

Instructor:Muhammad

Ahmad

EXPERIMENT 7

SETTING UP INTERRUPTS FOR REAL TIME INTERACTION WITH

TM4C123GH6PM

EXERCISE -1:

Implement a traffic control system. There is a single Traffic light whose lights turn on after every second. When traffic flow is less, after midnight, SW1 is pressed and yellow lights starts to blink with 1KHz rate. Another scenario is when traffic police take over and turn off the traffic light. This scenario is triggered by pressing SW2. Write interrupt driven code for this problem. In order to go back to normal mode from any scenario, same switch is pressed again. Both switches are at the same priority level.

CODE:
#include <stdint.h>

#include"C:\ti\TivaWare_C_Series-2.1.4.178\inc\tm4c123gh6pm.h"

// Modes

#define NORMAL_MODE   0

#define MIDNIGHT_MODE 1

#define POLICE_MODE   2

volatile uint8_t mode = NORMAL_MODE;

// Function prototypes

void PortF_Init(void);

void GPIOPortF_Handler(void);

void DelayMs(uint32_t ms);

void PortF_Init(void) {

SYSCTL_RCGCGPIO_R |= 0x20;        // Enable clock for Port F

while((SYSCTL_PRGPIO_R & 0x20)==0);

// Unlock PF0 (SW2)

GPIO_PORTF_LOCK_R = 0x4C4F434B;   // Unlock key

GPIO_PORTF_CR_R |= 0x01;          // Commit PF0

GPIO_PORTF_DIR_R |= 0x0E;         // PF1, PF2, PF3 as outputs (LEDs)

GPIO_PORTF_DIR_R &= ~0x11;        // PF0, PF4 as inputs (SW2, SW1)

GPIO_PORTF_DEN_R |= 0x1F;         // Enable digital function

GPIO_PORTF_PUR_R |= 0x11;         // Enable pull-up resistors for switches

// Configure interrupts

GPIO_PORTF_IS_R &= ~0x11;         // Edge sensitive

GPIO_PORTF_IBE_R &= ~0x11;        // Not both edges

GPIO_PORTF_IEV_R &= ~0x11;        // Falling edge

GPIO_PORTF_ICR_R = 0x11;          // Clear any prior interrupt

GPIO_PORTF_IM_R |= 0x11;          // Arm interrupts on PF0, PF4

NVIC_EN0_R |= (1<<30);            // Enable IRQ30 (Port F) in NVIC

}

// Interrupt Service Routine

void GPIOPortF_Handler(void) {

// SW2 (PF0)

if(GPIO_PORTF_RIS_R & 0x01) {

GPIO_PORTF_ICR_R = 0x01;    // Clear PF0 interrupt

if(mode == POLICE_MODE) {

mode = NORMAL_MODE;

} else {

mode = POLICE_MODE;

}

}

// SW1 (PF4)

if(GPIO_PORTF_RIS_R & 0x10) {

GPIO_PORTF_ICR_R = 0x10;    // Clear PF4 interrupt

if(mode == MIDNIGHT_MODE) {

mode = NORMAL_MODE;

} else {

mode = MIDNIGHT_MODE;

}

}

}

// Simple delay

void DelayMs(uint32_t ms) {

volatile uint32_t count;

while(ms--) {

count = 16000; // ~1ms at 16MHz

while(count--) {}

}

}

int main(void) {

PortF_Init();

while(1) {

if(mode == NORMAL_MODE) {

GPIO_PORTF_DATA_R = 0x02; // Red

DelayMs(100);             // Faster blink (0.5s)

GPIO_PORTF_DATA_R = 0x04; // Blue

DelayMs(100);

GPIO_PORTF_DATA_R = 0x08; // Green

DelayMs(100);

}

else if(mode == MIDNIGHT_MODE) {

GPIO_PORTF_DATA_R ^= 0x0A; // Toggle LEDs quickly

DelayMs(10);                // ~1kHz blink

}

else if(mode == POLICE_MODE) {

GPIO_PORTF_DATA_R &= ~0x0E;  // All LEDs off

}

}

}

EXERCISE -2:

Modify code such that if both switches are pressed at the same time, interrupt of SW2 is handled

first.

Ans

These two lines were added to set switch 2 priority higher then switch 1.

NVIC_PRI7_R = (NVIC_PRI7_R & 0xFF00FFFF) | (0x20 << 21); // PF0 priority = 1

NVIC_PRI7_R = (NVIC_PRI7_R & 0x00FFFFFF) | (0x40 << 29); // PF4 priority = 2