EXPERIMENT 8

From : M Husnain Khalid
22L:6286

To : Muhammad Ahmad

SENSOR INTERFACING USING ANALOG TO

DIGITAL CONVERTER

Exercise 1

Implement a voltmeter using TM4C using its ADC module. Read input voltage from any of its

channels and send its value (up to 2 decimal places) to PC via UART0. In order to test your code, implement a simple series resistive circuit on breadboard and read voltages of each node. Compare your results of your code with theoretical calculations.

Ans

Testing the Voltmeter Circuit

```

Use any two resistors (e.g., R1 = 10kO, R2 = 10kO). The voltage at Node A should be:

```

V_A = 3.3 × R2/(R1+R2) = 3.3 × 10k/20k = 1.65V

CODE:

Baud rate = 115200, system clock = 16MHz

ClkDiv = 16MHz / (16 × 115200) = 8.68

IBRD = 8, FBRD = 0.68 × 64 + 0.5 = 44

#include <stdint.h>

#include <TM4C123GH6PM.h>

void UART0_Init(void) {

SYSCTL_RCGCUART_R |= 0x01;       // Enable UART0 clock

SYSCTL_RCGCGPIO_R |= 0x01;       // Enable Port A clock

UART0_CTL_R &= ~0x01;            // Disable UART0 before configuring

UART0_IBRD_R = 8;

UART0_FBRD_R = 44;

UART0_LCRH_R = 0x60;             // 8-bit, no parity, 1 stop bit, no FIFO

UART0_CTL_R  = 0x301;            // Enable UART, TX, RX

GPIO_PORTA_AFSEL_R |= 0x03;      // PA0, PA1 ? alternate function

GPIO_PORTA_PCTL_R  |= 0x11;      // UART function for PA0, PA1

GPIO_PORTA_DEN_R   |= 0x03;      // Enable digital on PA0, PA1

GPIO_PORTA_AMSEL_R &= ~0x03;     // Disable analog on PA0, PA1

}

void UART0_SendChar(char c) {

while (UART0_FR_R & 0x20);       // Wait if transmit FIFO is full

UART0_DR_R = c;

}

void UART0_SendString(char *str) {

while (*str) {

UART0_SendChar(*str++);

}

}

void UART0_SendInt(int num) {

char buf[10];

int i = 0;

if (num == 0) {

UART0_SendChar('0');

return;

}

// Build digits in reverse

while (num > 0) {

buf[i++] = '0' + (num % 10);

num /= 10;

}

// Send in correct order

while (i > 0) {

UART0_SendChar(buf[--i]);

}

}

void ADC0_Init(void) {

SYSCTL_RCGCGPIO_R |= 0x10;

while ((SYSCTL_PRGPIO_R & 0x10) == 0); // Wait for it to be ready

GPIO_PORTE_DIR_R  &= ~0x08;     // PE3 = input (bit 3 = 0)

GPIO_PORTE_AFSEL_R |= 0x08;

GPIO_PORTE_DEN_R  &= ~0x08;

GPIO_PORTE_AMSEL_R |= 0x08;

SYSCTL_RCGCADC_R |= 0x01;

while ((SYSCTL_PRADC_R & 0x01) == 0); // Wait for ADC to be ready

ADC0_ACTSS_R &= ~0x08;          // Clear bit 3 (ASEN3)

ADC0_EMUX_R &= ~0xF000;         // Bits 15-12 = 0000 ? software trigger

ADC0_SSMUX3_R = 0;              // Channel 0 = AIN0 = PE3

ADC0_SSCTL3_R = 0x06;           // Bit1=END (last sample), Bit2=IE (interrupt enable)

ADC0_IM_R &= ~0x08;

ADC0_ACTSS_R |= 0x08;

}

uint32_t ADC0_Read(void) {

uint32_t result;

ADC0_PSSI_R = 0x08;             // Bit 3 = start SS3

while ((ADC0_RIS_R & 0x08) == 0); // Wait for interrupt raw status (SS3 done)

result = ADC0_SSFIFO3_R & 0xFFF; // Mask lower 12 bits

ADC0_ISC_R = 0x08;

return result;

}

uint32_t ADC_ToVoltage_x100(uint32_t adcVal) {

return (adcVal * 330) / 4095;

}

int main(void) {

uint32_t adcResult;

uint32_t voltage_x100; // e.g., 165 means 1.65V

uint32_t whole, decimal;

UART0_Init();

ADC0_Init();

UART0_SendString("Voltmeter Ready!\r\n");

while (1) {

adcResult = ADC0_Read();

voltage_x100 = ADC_ToVoltage_x100(adcResult);

whole   = voltage_x100 / 100;

decimal = voltage_x100 % 100;

UART0_SendString("Voltage = ");

UART0_SendInt(whole);

UART0_SendChar('.');

if (decimal < 10) UART0_SendChar('0');

UART0_SendInt(decimal);

UART0_SendString(" V\r\n");

volatile int d;

for (d = 0; d < 1000000; d++);

}

}

```

```