EXPERIMENT 9

USING PULSE WIDTH MODULATION FOR

PRECISION CONTROL OF EXTERNAL MODULES

From:Muhammad Husnain Khalid

Roll No:22l-6286

Class:8A2

To: Muhammad Ahmad

CODE:

#include <stdint.h>

#include "tm4c123gh6pm.h"

unsigned int notes[] = {262, 262, 294, 262, 349, 330, 262, 262, 294, 262, 392,

349, 262, 262, 523, 440, 349, 330, 294, 466, 466, 440,

349, 392, 349};

unsigned short interval[] = {4, 4, 8, 8, 8, 10, 4, 4, 8, 8, 8, 10, 4, 4, 8, 8, 8,

8, 8, 4, 4, 8, 8, 8, 12};

#define PWM_CLK  8000000

void PWM_Init(void);

void PWM_SetNote(uint32_t freq);

void delay_ms(uint32_t ms);

int main(void) {

int i;

PWM_Init();

while(1) {

for(i = 0; i < 25; i++) {

PWM_SetNote(notes[i]);

PWM1_ENABLE_R |= 0x40;

delay_ms(interval[i] * 100);

PWM1_ENABLE_R &= ~0x40;

delay_ms(50);

}

delay_ms(2000);

}

}

void PWM_Init(void) {

volatile uint32_t delay;

SYSCTL_RCGCPWM_R  |= 0x02;

SYSCTL_RCGCGPIO_R |= 0x20;

delay = SYSCTL_RCGCGPIO_R;

SYSCTL_RCC_R = (SYSCTL_RCC_R & ~0x000E0000) | 0x00100000;

GPIO_PORTF_AFSEL_R |= 0x04;

GPIO_PORTF_PCTL_R   = (GPIO_PORTF_PCTL_R & ~0x00000F00) | 0x00000500;

GPIO_PORTF_DEN_R   |= 0x04;

GPIO_PORTF_AMSEL_R &= ~0x04;

PWM1_3_CTL_R  = 0;

PWM1_3_GENA_R = 0x8C;

PWM1_3_LOAD_R = (PWM_CLK / 262) - 1;

PWM1_3_CMPA_R = PWM1_3_LOAD_R / 2;

PWM1_3_CTL_R  = 0x01;

PWM1_ENABLE_R &= ~0x40;

}

void PWM_SetNote(uint32_t freq) {

uint32_t load = (PWM_CLK / freq) - 1;

PWM1_3_LOAD_R = load;

PWM1_3_CMPA_R = load / 2;

}

void delay_ms(uint32_t ms) {

uint32_t i, j;

for(i = 0; i < ms; i++)

for(j = 0; j < 3180; j++);

}

Description

In PWM_Init we enable clocks for PWM1 and Port F, then set the PWM clock to 8MHz by dividing the system clock by 2 through the RCC register. PF2 is configured as alternate function M1PWM6 by setting AFSEL and writing 5 into its PCTL field. Generator 3 is disabled first while we write GENA=0x8C which tells it to go HIGH when the counter hits LOAD and LOW when it hits CMPA, then we start the generator but keep the output pin disabled.

PWM_SetNote just calculates LOAD as (8MHz / freq) - 1 and sets CMPA to half of that to get 50% duty cycle.

In main we loop through all 25 notes, each time calling PWM_SetNote to set the frequency, enabling the output, waiting for interval[i] × 100ms, then disabling the output and adding a 50ms gap so adjacent identical notes don't blend together. After all 25 notes there's a 2 second pause before the song repeats.