EXPERIMENT 10

DESIGN AND IMPLEMENTATION OF AN INTEGRATED SMART POWER MONITORING STATION

OBJECTIVE:

- To design a multi-node embedded system solution for a real-world problem using various peripherals including GPIO, UART, SSI, ADC and PWM.

- To demonstrate proficiency in C language constructs and bit-level hardware manipulation.

EQUIPMENT REQUIRED:

- 1x TM4C123G LaunchPad.

- 1x BOOSTXL-EDUMKII BoosterPack.

- Breadboard and resistors (for the voltmeter test circuit).

- Keil µVision 4 and Tera Term

PROBLEM STATEMENT: You are to design a "Smart Power Monitoring Station" for an industrial workstation. In this scenario, your system must monitor the voltage levels of an external circuit (simulating a power rail) while providing a user interface for configuration and emergency alerts.

Since you have already mastered ADC-based Voltmeter implementation, your system must perform the following integrated tasks using only polling logic:

- Precision Voltage Monitoring (ADC):

  - The system must use the ADC0 module to monitor an analog voltage connected to an available AIN channel (e.g., AIN0/PE3 or AIN1/PE2).

  - Using the ADC conversion formula (), the system must calculate the actual voltage level (0.00V to 3.30V).

  - Logic: Continuously poll the ADCRIS register to detect when a conversion is complete before reading the ADCSSFIFOn.

- Interactive Terminal Interface (UART):

  - Configure UART0 at 115200 baud to communicate with a PC.

  - The system must display a "Live Telemetry" screen. When the user types the character 'V', the current calculated voltage must be printed to the terminal with two decimal places.

  - If the voltage falls below a "Critical Low" threshold (e.g., 1.0V) or exceeds a "Critical High" threshold (e.g., 3.0V), the system must automatically send a "VOLTAGE OUT OF RANGE" warning message to the PC.

- Manual Control and Visual Status (GPIO):

  - System Arming: Use the BoosterPack Pushbutton S1 (J4.33) to "Arm" or "Disarm" the alarm system.

  - Polling: Continuously check the status of the buttons by reading the GPIODATA register (using bit-masking).

  - Visual Status: Use the RGB LED on the BoosterPack. If the system is "Armed," the Green LED should be on. If a voltage fault is detected, the Red LED should blink using a software delay.

TASK DELIVERABLES:

- Circuit Setup: A breadboard circuit (e.g., a resistive voltage divider or potentiometer) connected to the Tiva C AIN pin to provide a variable test voltage.

- Implementation Report:

  - Flowchart: A detailed diagram showing how the main loop polls the ADC, then the UART, then the GPIO buttons.

  - Source Code: Fully commented C code implementing the requirements.

- Demonstration: Show the system reporting voltages to Tera Term and triggering the buzzer/LEDs based on the analog input level.